Baseline Hardware Specifications Version 2.0

IMPORTANT – These tables serve as a broad guidelines on the core baseline specifications that normally define the various types of equipment to be procured. Contracting Authorities are at liberty to specify further specifications as they deem necessary and shall define the actual requirements to best suit their procurement needs including adding any additional requirements to the hardware specifications. Please also refer to the Guidance Notes (MITA-PRO-DPS 056_25 Guidance Notes-v2.0) for any additional technical requirements or specifications that might need to be included. When drafting the technical requirements, special attention should be given to the performance characteristics of the hardware being sought. These performance characteristics could be achieved through a variety of technologies (for example, in the case of leasing of multifunction printers, by machines using laser or inkjet technology). The specifications should therefore, unless otherwise necessitated by any specific justification, allow for competition between differing technologies to the greatest extent possible.

| Requirement in these cells: |  | to be defined. |
| --- | --- | --- |

- Standalone Printer (Colour / Monochrome)

Purpose: Designed for low-volume, everyday personal use.

Features: Compact size, affordable, easy to set up. Typically inkjet or small laser models. Often include printing features, Wi-Fi connectivity, and basic scanning/copying.

Print Quality: Good for daily documents and occasional photos. Maximum resolution is usually sufficient for text and casual images, but not for professional graphics.

Typical Use Cases:

- Executive office with around a maximum of 5 users.

- Occasional scanning or copying.

- Small business needs, such as invoices and labels.

| # | Description | Requirement |
| --- | --- | --- |
| 1 | Technology | To define (if necessitated by any specific justification, N/A if there is no particular technology requirement) |
| 2 | Print Speed | To define [Colour: >= 20 ppm] [Monochrome: >=25-35 ppm] |
| 3 | Colour/Monochrome | To define |
| 4 | Duplex printing | Yes |
| 5 | Resolution | To define [600 X 600 DPI] [1200 X 1200 DPI] |
| 6 | Memory | >=128Mb |
| 7 | Paper input capacity | To Define [250 sheet paper tray / 500 sheet paper tray / multiple trays with additional bypass tray if looking for higher end printers] |
| 8 | Paper Size | To define [A4, A3] |
| 9 | Paper thickness handling range | To define [>=70gms, (75-80 is recommended as a minimum), optional 100gms or over for specific letterheads/certificates etc.] |
| 10 | Paper type handling | To define [uncoated/plain paper, matte, gloss, + optional (labels, envelopes, letterheads)] |
| 11 | Printer Language | PCL 6, Postscript (optional if printing from Apple Mac systems is being considered) |
| 12 | Monthly Duty Cycling | To define [Minimum 30,000 pages] |
| 13 | Standard Ports | USB 2.0, Gigabit Ethernet (100/1000) |
| 14 | Display | To Define [yes/no] |
| 15 | Wireless Connectivity | To Define [yes/no; Wifi/Bluetooth] |

2. Standard Multifunction Printer

Purpose: Built for moderate to high-volume printing in a shared work environment.

Features: Larger, faster, and more robust than standalone printers. Often multifunctional (print, scan, copy, fax). Support for network connectivity, automatic document feeders, duplex printing, and sometimes finishing options like stapling.

Print Quality: Excellent for text and business graphics; suitable for presentations, contracts, and marketing materials, but not as high-end as production printers for intricate graphics.

Typical Use Cases:

- Printing contracts, invoices, reports, and presentations.

- Scanning and copying documents for digital archiving.

- Producing internal marketing materials, labels, or compliance documentation.

| # | Description | Requirement |
| --- | --- | --- |
| 1 | Technology | To define (if necessitated by any specific justification, N/A if there is no particular technology requirement) |
| 2 | Print Speed | [Colour: >= 20 ppm] [Monochrome: >=35 ppm] |
| 3 | Colour/Monochrome | To define [Colour/Monochrome only] |
| 4 | Duplex printing | Yes |
| 5 | Resolution | To define [600 X 600 DPI] [1200 X 1200 DPI] |
| 6 | Memory | >=512Mb |
| 8 | Internal Hard Disk Space | >=100Gb |
| 9 | Paper input capacity | To Define [Minimum 250 sheet paper tray; 50 sheet ADF tray input capacity] |
| 10 | Paper Size | To define [A4, A3] |
| 11 | Paper thickness handling range | To define [>=70gms, (75-80 is recommended as a minimum), optional 100gms or over for specific letterheads/certificates etc.] |
| 12 | Paper type handling | To define [uncoated/plain paper, matte, gloss, + optional (labels, envelopes, letterheads)] |
| 13 | Printer Language | PCL 6, Postscript (optional if printing from Apple Mac systems is being considered) |
| 14 | Scanning | To define Flatbed only [and/or] Automatic Document Feeder (ADF) |
| 15 | Maximum Scan Area | A3 |
| 16 | Automatic Duplex Scanning | Yes |
| 17 | Scan-to-Email using SMTP Server | Yes |
| 18 | Automatic Duplex Photocopying | Yes |
| 19 | Monthly Duty Cycling | To define [Minimum 70,000 pages] |
| 20 | User Interface | Touch screen display |
| 21 | Standard Ports | USB 2.0, Gigabit Ethernet (100/1000) |
| 22 | Authenticated Printing: - Password protection prior to printing, delay printing until password is re-entered at device - Support for Active Directory user authentication - Encryption of Internal Hard Disk Ability to wipe data on Internal Hard Disk | To define [yes/no] Strongly recommended to be asked for especially if sensitive documents are expected to be printed. |
| 23 | Compatibility with Printing Management Solution, e.g. Cloud Printing. (If procuring multiple printers that will be connected centrally to a managed printing solution, it is recommended to cater for the necessary service(s), including hosting, licenses, network access and security requirements (Refer to GMICT policies for guidelines). Such solutions are to be procured separately outside of this DPS.) | To define |

- Heavy-Duty Multifunction Printer

Purpose: Engineered for high-volume, and high-speed output.

Features: Large physical footprint, advanced paper handling, and finishing options (e.g., binding, folding, stapling). Superior color management, media flexibility (handles heavy cardstock, glossy, or specialty papers), often in higher resolutions available.

Print Quality: Professional-grade, suitable for detailed graphics, vibrant colors, and precise color matching—critical for branding and marketing.

Typical Use Cases:

- Centralised printing, copying, and scanning for departments with high daily document output, document processing and archiving.

- High-impact visuals for marketing campaigns.

- Printing of course materials, exams and administrative documents in an Educational facility.

| # | Description | Requirement |
| --- | --- | --- |
| 1 | Technology | To define (if necessitated by any specific justification, N/A if there is no particular technology requirement) |
| 2 | Print Speed | >= 75 ppm |
| 3 | Colour/Monochrome | Colour |
| 4 | Duplex printing | Yes |
| 5 | Resolution | To define [1200DPI] [2400DPI] |
| 6 | Memory | >=2Gb |
| 7 | Internal Hard Disk Space | >=100Gb |
| 8 | Paper input capacity | To Define [Minimum 500 sheet paper tray; /multiple trays with additional bypass tray] |
| 9 | Paper Size | To define [A4, A3] |
| 10 | Paper thickness handling range | To define [>=70gms, (75-80 is recommended as a minimum), optional 100gms or over for specific letterheads/certificates etc.] |
| 11 | Paper type handling | To define [uncoated/plain paper, matte, gloss, + optional (labels, envelopes, letterheads)] |
| 12 | Finishing | To define [Stapling, Punching, Folding] |
| 13 | Printer Language | PCL 6, Postscript (optional if printing from Apple Mac systems is being considered) |
| 14 | Scanning | Flatbed and Automatic Document Feeder (ADF) |
| 15 | Maximum Scan Area | A3 |
| 16 | Automatic Duplex Scanning | Yes |
| 17 | Scan-to-Email using SMTP Server | Yes |
| 18 | Secure Printing: - Password protection prior to printing, delay printing until password is re-entered at device - Support for Active Directory user authentication - Encryption of Internal Hard Disk - Ability to wipe data on Internal Hard Disk | Yes |
| 19 | Automatic Duplex Photocopying | Yes |
| 20 | Monthly Duty Cycling | To define [Minimum 100,000 pages] |
| 21 | User Interface | Touch screen display |
| 22 | Standard Ports | USB 2.0, Gigabit Ethernet |
| 23 | Compatibility with Printing Management Solution, e.g. Cloud Printing. (If procuring multiple printers that will be connected centrally to a managed printing solution, it is recommended to cater for the necessary service(s), including hosting, licenses, network access and security requirements (Refer to GMICT policies for guidelines). Such solutions are to be procured separately outside of this DPS.) | Required/Not Required |

4. 3D Printer

| # | Description | Requirement |
| --- | --- | --- |
| 1 | Technology | To define (if necessitated by any specific justification, N/A if there is no particular technology requirement)w |
| 2 | XY Resolution | To define [>=75μm] (“K” rating) |
| 3 | Layer Resolution (Z Resolution) | To define [>=10μm] (“K” rating) |
| 4 | Printing Area | To define [150 X 150 X 150 mm] |
| 5 | Nozzle Diameter | To define [0.4mm (expected lifetime 1,500 hours] |
| 6 | Multiple Spools / Printing heads (for Multicolour print) | To define |
| 7 | Minimum Print Speed | To define |
| 8 | Temperature range | To define |
| 9 | Print bed type | To define [Glass, Removable, Heated, manual/auto-leveling] |
| 10 | Calibration | Automatic |
| 11 | Maximum Print Volume (inch) | To define 6.1 X 3.7 X 7.1 (L X W X H) |
| 12 | Material | To define [Resin, PLA, ABS, PETG, Nylon etc] |
| 13 | XY Resolution Control | To define [Yes, No] |
| 14 | Safety elements | To define [Door Locks, print bed sensor, emergency stop, auto-shutdown; filament run-out sensors] |
| 15 | Software | To define [Generic/Dedicated] including Automatic slicing / Support] including file format support [STL, OBJ, STEP etc] |
| 16 | Connectivity | USB, Ethernet, Wi-Fi Resume print after power outage |

5. Scanner

| # | Description | Requirement |
| --- | --- | --- |
| 1 | Scanner type | To define [flatbed, book, production, handheld, sheetfed, photo] |
| 2 | Photoelectric Device | To define |
| 3 | Effective pixels | To define |
| 4 | Document Size | To define [A4; A3] |
| 5 | Scanning Resolution | To define |
| 6 | Scanning Speed | To define [Colour: Greyscale: ] |
| 7 | Output Resolution | To define |
|  | Image enhancements | To define [Deskew, Despeckle, Colour correction] |
| 8 | Daily Duty Cycle | >= 1,500 pages |
| 9 | Interface | USB 2.0, Gigabit Ethernet Port |
| 10 | Light Source | To define [RGB LED White LED, IR LED] |
| 11 | Software Included | To define [Scan Utility, PDF Utility, File Format support etc] |
| 12 | OCR capabilities | To define [Blank Page removal, searchable/editable document output / redactable sensitive data] |
| 13 | Security features | To define [Encryption/secure file handling] |